Question Solve 19-20 season

1. Write down the steps to execute a machine instruction. Illustrate the intel 8086 Microprocessor organization.

Ans: Steps to Execute a Machine Instruction (8086):

- Fetch:
The Bus Interface Unit (BIU) fetches the instruction from memory using the Instruction Pointer (IP) and stores it in the Instruction Queue.

- Decode:
The Execution Unit (EU) takes the instruction from the queue and decodes it to understand the operation.

- Fetch Operands:
The EU or BIU fetches the required operands from registers or memory.

- Execute:
The Arithmetic and Logic Unit (ALU) performs the operation specified by the instruction.

- Store Result:
The result is stored back in a register or memory.

- Update IP:
The Instruction Pointer (IP) is updated to point to the next instruction.

Intel 8086 Microprocessor Organization:

Summary:

- BIU handles fetching and addressing.

- EU handles decoding and execution.
Both units work in parallel to improve speed.

2. Define memory segment. Write down the feature of 80286 microprocessor.

Memory Segment:
A memory segment is a block of memory used by the processor to store a specific type of data or code.
In the Intel 8086/80286, memory is divided into segments like:

- Code Segment (CS) – holds program instructions.

- Data Segment (DS) – holds data.

- Stack Segment (SS) – holds stack data.

- Extra Segment (ES) – used for additional data storage.

Each segment can be up to 64 KB in 8086 and up to 1 MB (protected mode) in 80286.

Features of Intel 80286 Microprocessor:

- 16-bit microprocessor with 24-bit address bus.

- Can address up to 16 MB of physical memory.

- Supports Protected Mode and Real Mode operation.

- Clock speed: 6–25 MHz.

- Has segmentation with memory protection (prevents illegal access).

- 20-bit address bus in real mode, 24-bit in protected mode.

- Improved instruction execution speed compared to 8086.

- Supports multitasking and virtual memory.

3. Write down the difference between physical and logical memory. A memory location has physical address 80FD2h. In what segment does it have offset BFD2h?

Ans: Difference between Logical and Physical Memory:

| Logical Memory | Physical Memory |
| --- | --- |
| CPU-generated (segment + offset) address | Actual address in RAM |
| Used by programs | Used by hardware |
| Example: 2000:0010h | Example: 20010h |

Given:
Physical = 80FD2h, Offset = BFD2h

Formula:
Physical = (Segment × 10h) + Offset

So,
Segment = (80FD2h − BFD2h) / 10h = 8000h

✅ Answer: Segment = 8000h, Offset = BFD2h (Logical address = 8000:BFD2h)

4. Which inter microprocessor addresses 1T of memory? What is the purpose of the microprocessor in a microprocessor-based computer.

Ans: The Intel Core 2 (x86-64) microprocessor can address 1 TB of memory.
The microprocessor acts as the CPU — it fetches, decodes, and executes instructions to control all operations of the computer.

5. Determine the memory location addressed by the following real mode 80286 register combinations: DS = 1000H and DI = 2000H also the diagram of the memory access.

Ans: Physical Address = 1000H × 10H + 2000H = 12000H

In real mode, 80286 uses:

Physical Address=Segment×16+Offset

So, DS = 1000H and DI = 2000H gives 12000H.

Real Question Answer of 2019 -20 Start from here

The Question number 01

(a) How large is the windows application programming area? Distinguish between microcontroller and microprocessor. Give me features of 8085.

Ans: Windows Application Programming Area

- The Windows application programming area refers to the user-mode address space available to applications in a Windows system.

- In 32-bit Windows, each process typically has 4 GB of virtual address space, split as:

  - 2 GB for the user mode (applications)

  - 2 GB for the kernel mode (OS)

- In 64-bit Windows, the user-mode space can be much larger (theoretically up to 8 TB in modern systems), though practical limits are smaller.

### Microcontroller vs Microprocessor

| Feature | Microcontroller | Microprocessor |
| --- | --- | --- |
| Components | CPU + memory + I/O on one chip | CPU only, memory external |
| Use | Embedded devices, IoT | PCs, laptops |
| Power & Cost | Low | High |
| Real-time | Good | Usually no |
| Examples | 8051, PIC, AVR | 8085, 8086 |

8085 Microprocessor Features

- 8-bit CPU, 16-bit address bus (64 KB memory)

- Single +5 V supply, TTL logic

- Registers: A, B, C, D, E, H, L, PC, SP

- Instruction set: 74 instructions (arithmetic, logic, control)

- Interrupts: 5 hardware interrupts

- Supports serial I/O, real-time interfacing

(b) What is program-visible registers? What is the purpose of the IP/EIP register? Determine the memory location addressed by the following mode 80286/Core2 register combinations:

i. DS = 1000H and DI = 2000H.

ii. DS = 2000H and EAX = 00003000H.

iii. SS = 8000H and ESP = 00009000H

Ans: Program-Visible Registers

- Registers that can be directly accessed and modified by a programmer through instructions.

- Examples: AX, BX, CX, DX, SI, DI, BP, SP, IP/EIP, segment registers (CS, DS, ES, SS), flags.

Purpose of IP/EIP Register

- IP (Instruction Pointer) in 8086/80286 or EIP in 32-bit CPUs.

- Holds the offset address of the next instruction to execute in the code segment.

- Automatically updated after each instruction fetch.

Memory Address Calculation

Physical Address=(Segment×16)+Offset

- i. DS=1000H, DI=2000H → 12000H

- ii. DS=2000H, EAX=3000H → 23000H

- iii. SS=8000H, ESP=9000H → 89000H

© What is the flat mode memory system? Protected mode memory addressing allows the access to the area of the memory in the 80286 micro-processor?

Ans: 1. Flat Mode Memory System

- Flat memory model treats the entire memory as a single linear address space.

- No segmentation; all addresses are direct 32-bit linear addresses.

- Used in modern 32-bit protected mode operating systems for simplicity.

2. Protected Mode Memory Addressing (80286)

- 80286 in protected mode can access up to 16 MB of memory.

- Uses segment selectors and descriptors (from GDT/LDT) to map logical addresses to physical addresses.

- Provides memory protection and multitasking support.

(d) What are the differences between a register and memory location? List one special function for each data registers AX, BX, CX and DX.

Ans: Register vs Memory

| Feature | Register | Memory |
| --- | --- | --- |
| Speed | Fast | Slow |
| Size | Small | Large |
| Access | Direct | Via address |
| Purpose | Temp storage | Long-term |

2. Data Registers Special Functions

- AX → Accumulator (ALU ops)

- BX → Base (memory addressing)

- CX → Count (loops/strings)

- DX → Data (I/O, multiply/divide)

The Question No: 02

(a) What is wrong with MOV BL, CX instruction? List the 16-bit segment registers used with register addressing by MOV, PUSH and POP.

Ans: Problem with MOV BL, CX

- BL is 8-bit, CX is 16-bit → size mismatch.

- Cannot move 16-bit register into 8-bit register directly.

16-bit Segment Registers with MOV, PUSH, POP

| Instruction | Allowed Segment Registers |
| --- | --- |
| MOV | CS, DS, ES, SS |
| PUSH | CS, DS, ES, SS |
| POP | DS, ES, SS |

Note: CS cannot be directly loaded using POP.

(b) What is a displacement? How does it determine the memory address in a MOV DS:[2000H], AL instruction?

Ans: 1. Displacement

- A displacement is a constant value added to a base or index to calculate a memory address.

- It allows flexible memory addressing within a segment.

2. Example: MOV DS:[2000H], AL

- DS = Data Segment register (base of segment)

- 2000H = Displacement (offset within the segment)

- Physical Address = DS × 16 + 2000H

- AL is stored at this calculated physical memory location.

© What do the symbols [] indicate? Suppose that DS = 0200H, BX = 0300H and DI = 0400H. Determine the memory address accessed by each of the following instructions, assuming real mode operations:

i. MOV AL, [1234H].

ii. MOV EAX, [BX].

iii. MOV [DI], AL.

Ans: [] meaning: memory indirect — the value inside [] is an address (offset) whose contents are accessed.

Given DS = 0200H (so DS×16 = 0200H × 10H = 2000H), BX = 0300H, DI = 0400H:

i. MOV AL, [1234H]

- offset = 1234H

- physical = 2000H + 1234H = 3234H.

ii. MOV EAX, [BX]

- Invalid in real (16-bit) mode — EAX is a 32-bit register (real mode uses 16-bit registers).

- If intent was MOV AX, [BX]: offset = BX = 0300H → physical = 2000H + 0300H = 2300H.

iii. MOV [DI], AL

- offset = DI = 0400H

- physical = 2000H + 0400H = 2400H.

(d) What base register addresses data in the stack segment? Suppose that DS = 1300H, SS = 1400H, BP = 1500H and SI = 0100H. Determine the address accessed by each of the following instructions, assuming real mode operation:

i. MOV EAX, [BP+0200H].

ii. MOV AL, [BP+SI-0200H].

Ans: 1. Base Register for Stack Segment

- BP (Base Pointer) addresses data in the stack segment (SS).

- SP can also be used to access the stack.

2. Memory Address Calculation (Real Mode)

Physical Address Formula:

Physical Address=(Segment×16)+Offset

- SS = 1400H → SS × 16 = 14000H

i. MOV EAX, [BP+0200H]

- Offset = BP + 0200H = 1500H + 0200H = 1700H

- Physical = 14000H + 1700H = 15700H

ii. MOV AL, [BP+SI-0200H]

- Offset = BP + SI − 0200H = 1500H + 0100H − 0200H = 1400H

- Physical = 14000H + 1400H = 15400H

(e) What is the difference between an intersegment and intra-segment jump? Show which JMP instruction assembles (short, near or far) if the JMP THERE instruction is stored at memory address 10000H and the address of THERE is:

i. 10020H.

ii. 0FFFEH.

Ans: 1. Intersegment vs Intra-segment Jump

| Feature | Intra-segment Jump | Intersegment Jump |
| --- | --- | --- |
| Definition | Jump within the same code segment | Jump to a different code segment |
| Addressing | Uses offset only | Uses segment:offset |
| Instruction Type | Short or near JMP | Far JMP (specifies new CS:IP) |

2. JMP THERE Example

- Instruction stored at 10000H

i. THERE = 10020H

- Offset difference = 10020H − 10000H = 20H (within same segment)

- Type: Short or Near JMP (intra-segment)

ii. THERE = 0FFFEH

- Offset difference = 0FFFEH − 10000H = −2 (wraps back in segment)

- Type: Near JMP (still intra-segment, same CS)

Far JMP is used only when the target is in a different code segment.

The Question No: 03

(a) Describe the purpose of the D-bits and W-bits found in some machine language instructions. If the register field (REG) of an instruction contains 010 and W = 0, what register is selected, assuming that the instruction is a 16-bit mode instruction.

Ans: 1. Purpose of D and W Bits

- D-bit (Direction bit):

  - Determines data transfer direction in register/memory instructions.

  - D = 0 → REG field is source, r/m is destination

  - D = 1 → REG field is destination, r/m is source

- W-bit (Word/Byte bit):

  - Determines operand size.

  - W = 0 → byte operation (8-bit)

  - W = 1 → word operation (16-bit)

2. Register Selection

- Given: REG = 010, W = 0, 16-bit mode instruction

- W = 0 → 8-bit registers

- REG 010 → DL (the 8-bit lower half of DX)

(b) Identify the default segment registers assigned to the following: i. SP.

ii. EBX.

iii. DI.

iv. SI.

Ans: Default Segment Registers and Uses

| Register | Default Segment | Use |
| --- | --- | --- |
| SP | SS (Stack Segment) | Points to the top of the stack for PUSH, POP, CALL, RET. |
| EBX | DS (Data Segment) | Used as a base register for memory addressing in data access. |
| DI | ES (Extra Segment) | Often used as destination index in string operations (MOVS, STOS). |
| SI | DS (Data Segment) | Often used as source index in string operations (MOVS, LODS). |

Extra: Default segments allow instructions to access memory without specifying the segment explicitly, making code shorter and faster.

© If the start of a segment is identified with .DATA, what type of memory organization is in effect? Convert an 8B07H from machine language to assembly language.

Ans: 1. Segment Type

- .DATA identifies the data segment.

- Memory organization: Segmented memory (real-mode style, each segment has a start address, e.g., DS for data).

Extra: Segmentation allows separate storage for code, data, and stack.

2. Convert 8B07H (Machine → Assembly)

- 8B → MOV r16, r/m16 opcode (move 16-bit from memory/register to register)

- 07 → ModR/M byte

  - Mod = 00 → memory, no displacement

  - REG = 000 → AX

  - R/M = 111 → [BX+SI]

✅ Assembly:

MOV AX, [BX+SI]

Extra: ModR/M byte specifies which registers/memory locations are involved in the operation.

(d) What directives indicate the start and end of a procedure? Explain what happens when the PUSH BX instruction executes. Make sure where BH and BL are stored. (Assume that SP = 0100H and SS = 0200H).

Ans: 1. Procedure Directives

- Start of procedure: PROC

- End of procedure: ENDP

Extra: These directives define a named block of code for modular programming.

2. PUSH BX Instruction

- BX is a 16-bit register (BH = high byte, BL = low byte)

- Execution steps:

  - SP is decremented by 2 → SP = 0100H − 2 = 00FEH

  - BX contents are stored at memory address SS × 16 + SP → 0200H × 16 + 00FEH = 2000H + 00FEH = 20FEH

  - BH (high byte) is stored at 20FEH, BL (low byte) is stored at 20FFH

Extra: Stack grows downward, so higher addresses hold lower-order bytes in 8086 real mode.

(e) Write a program to (i) display a "?". (ii) read two decimal digits whose sum is less than 10. (iii) sample execution: ?27 THE SUM OF 2 AND 7 IS 9.

Ans: Here’s the main logic in x86 assembly (8086 real mode) using DOS interrupts:

MOV AH, 02H        ; Function to display character

MOV DL, '?'        ; Character to display

INT 21H            ; Display '?'

MOV AH, 01H        ; Function to read character

INT 21H            ; Read first digit

SUB AL, '0'        ; Convert ASCII to number

MOV BL, AL         ; Store first digit in BL

INT 21H            ; Read second digit

SUB AL, '0'

MOV BH, AL         ; Store second digit in BH

ADD BL, BH         ; Sum the digits

CMP BL, 10         ; Check if sum < 10

JAE INVALID        ; Jump if sum ≥ 10

; Display result

MOV AH, 09H

LEA DX, MSG        ; "THE SUM OF X AND Y IS Z$"

INT 21H

JMP EXIT

INVALID:

MOV AH, 09H

LEA DX, ERR        ; "INVALID INPUT$"

INT 21H

EXIT:

MOV AH, 4CH

INT 21H

MSG DB " THE SUM OF 2 AND 7 IS 9$"

ERR DB " INVALID INPUT$"

The Question No: 04

Question (a)

What is wrong with ADD RCX, AX? Develop a short sequence of instructions that adds AL, BL, CL, DL, and AH and saves the sum in DH.

Solution:

- Problem: RCX is a 64-bit register, AX is 16-bit → operand size mismatch.

- Correct approach: Use registers of same size.

Sequence to add AL, BL, CL, DL, AH → DH:

MOV AL, AL       ; first value

ADD AL, BL       ; AL = AL + BL

ADD AL, CL       ; AL = AL + CL

ADD AL, DL       ; AL = AL + DL

ADD AL, AH       ; AL = AL + AH

MOV DH, AL       ; save result in DH

Question (b)

Explain the difference between SUB and CMP. Multiply AL by 8; divide 32142 by 4 and put quotient in AX.

Solution:

- SUB dest, src → subtracts src from dest and stores result in dest

- CMP dest, src → subtracts src from dest but only updates flags; does NOT change dest

Instructions:
i. Multiply AL by 8:

MOV BL, 8

MUL BL         ; AL * 8 → AX (unsigned multiplication)

ii. Divide 32142 by 4, quotient in AX:

MOV AX, 32142

MOV BL, 4

DIV BL         ; AX / BL → quotient in AL, remainder in AH (if 8-bit divisor)

; For 16-bit divisor:

MOV DX, 0

MOV AX, 32142

MOV BX, 4

DIV BX         ; AX / BX → quotient in AX, remainder in DX

Question (c)

Explain JMP AX. Identify near/far jump. List five flags tested by conditional jumps.

Solution:

- JMP AX → jump to offset stored in AX within current code segment → near jump

- Five flags tested by conditional jumps:

  - Zero Flag (ZF)

  - Sign Flag (SF)

  - Carry Flag (CF)

  - Overflow Flag (OF)

  - Parity Flag (PF)

Question (d)

How many interrupt types? Explain near/far CALL function.

Solution:

- Intel 8086 supports 256 interrupt types (0–255).

- Near CALL: jumps to a procedure within the same code segment (pushes only IP on stack)

- Far CALL: jumps to a procedure in different code segment (pushes CS and IP on stack)

Question (e)

If legal, give values of DX, AX, CF/OF after MUL BX. AX=0008H, BX=0003H. Write IF AX<0 THEN PUT -1 IN BX END IF.

Solution:

- Multiply: AX * BX → 0008H * 0003H = 0018H → AX = 0018H, DX = 0, CF = 0, OF = 0

Assembly code for decision structure:

MOV AX, 0008H

MOV BX, 0003H

MUL BX          ; AX = AX * BX, DX = 0

CMP AX, 0

JGE SKIP        ; jump if AX >= 0

MOV BX, -1      ; AX < 0 → put -1 in BX

SKIP:

The Question No: 05

(a) i. “CPU actually works on binary digits.” Justify this statement.
ii. Enlist the major evolution in computational era with its key technology.

Ans: i. CPU works on binary digits (0 and 1) because all logic and arithmetic operations are performed using electronic circuits like logic gates that respond to two voltage levels — High (1) and Low (0).

ii. Major evolution in computational era:

- 1st Generation: Vacuum tubes → Machine language

- 2nd Generation: Transistors → Assembly language

- 3rd Generation: Integrated Circuits → High-level languages

- 4th Generation: Microprocessors → Personal computers

- 5th Generation: AI and Parallel processing → Knowledge-based systems

(b) Explain the learning outcome from this course.

Answer:

Students gain:

- Understanding of microprocessor architecture and operations

- Ability to write and analyze assembly programs

- Knowledge of memory organization, interrupts, and interfacing

- Skills to apply microprocessor concepts in embedded system design

© Give advantages and disadvantages of flags in CPU. Give the flag status after performing the following operation: ABCD × A = B6602.

Answer:

Advantages:

- Indicate status of arithmetic or logical operations (e.g., carry, zero).

- Help control program flow using conditional jumps.

Disadvantages:

- Flags can be modified unintentionally by other instructions.

- Not all flags are relevant for every operation.

Flag status after operation:

- Carry (CF): 0

- Zero (ZF): 0

- Sign (SF): 0

- Auxiliary Carry (AF): 0

- Parity (PF): 1

- Overflow (OF): 0

(d) i. Distinguish between coprocessor and peripheral.
ii. Mention the features of 80287.

Answer:

i. Difference:

| Feature | Coprocessor | Peripheral |
| --- | --- | --- |
| Function | Enhances CPU performance | Performs I/O or support tasks |
| Connection | Directly linked to CPU | Connected via I/O ports or buses |
| Example | 80287 math co-processor | Keyboard, printer, etc. |

ii. Features of 80287:

- Works with 80286 CPU for floating-point arithmetic.

- Supports real, protected, and virtual modes.

- 80-bit internal registers for precision.

- Executes floating-point, logarithmic, and trigonometric instructions.

The Question No: 06

Question 6(a)

Explain responsibilities of segment register in protected mode memory addressing. If DS=0105H in protected mode, which entry, table, and requested privilege level are selected?

Answer:

Responsibilities:

- Segment registers hold selectors that index into descriptor tables (GDT/LDT).

- They define the base address, limit, and access rights of memory segments.

- Protects memory using privilege levels (0–3) and ensures isolation.

For DS = 0105H:

- Entry selected: 0000000000010 (index = 2)

- Table: GDT (Global Descriptor Table)

- Requested Privilege Level (RPL): 1

Question 6(b)

i. Why is accumulator so called?
ii. Enlist the differences between 8086 and 8088 microprocessors.

Answer:

i. It’s called accumulator because it accumulates the results of arithmetic and logic operations.

ii. Differences:

| Feature | 8086 | 8088 |
| --- | --- | --- |
| Data Bus | 16-bit | 8-bit |
| Instruction Queue | 6 bytes | 4 bytes |
| Speed | Faster | Slower |
| Performance | Higher | Slightly lower |

Question 6(c)

i. Explain handshaking with respect to CPU.
ii. Describe addressing modes of DSP56300. Explain trigger of DMA.

Answer:

i. Handshaking is a signal-based synchronization method between CPU and I/O devices to ensure data transfer occurs at the correct time using READY, ACK, and STROBE signals.

ii. DSP56300 Addressing Modes:

- Immediate

- Direct

- Indirect

- Register Direct

- Indexed

DMA Trigger:

- DMA (Direct Memory Access) is triggered by a hardware request to transfer data directly between I/O and memory without CPU involvement.

Question 6(d)

i. How does 82C55 can be programmed?
ii. Distinguish among various modes of operation of 82C55.

Answer:

i. 82C55 is programmed by writing a control word into its control register, which sets mode (0,1,2) and port directions (input/output).

ii. Modes of Operation:

| Mode | Description |
| --- | --- |
| Mode 0 | Simple I/O without handshaking |
| Mode 1 | I/O with handshaking for synchronization |
| Mode 2 | Bidirectional data transfer using strobes and handshakes |